1. Classification of Surveying

Surveying is classified on the basis of purpose, place, instrument used, and accuracy.

(a) Based on Accuracy

- Plane Surveying –

  - Earth’s surface considered flat.

  - Used for small areas (error due to curvature negligible).

  - Applications: Roads, canals, buildings, bridges, boundaries.

  - Assumptions:

    - Level lines are straight.

    - Meridians are parallel.

    - Angles are plane angles.

- Geodetic Surveying –

  - Curvature of the Earth considered.

  - Used for large areas (national mapping, control networks).

  - Angles are spherical; measurements corrected for curvature.

  - Conducted by organizations like the Survey of India.

(b) Based on Purpose

- Topographical Survey: Determines natural features (hills, valleys, rivers).

- Cadastral Survey: Establishes property boundaries and land ownership.

- Engineering Survey: For design and construction of structures, roads, bridges.

- Hydrographic Survey: Determines features of bodies of water.

- Astronomical Survey: Determines absolute positions of points using celestial observations.

- Geological / Mine Survey: Determines strata, faults, and underground features.

- Archaeological Survey: For excavation and locating ancient structures.

- Route Survey: For alignment of roads, pipelines, canals, or railways.

(c) Based on Instruments Used

- Chain Surveying – Uses chain or tape for linear measurement.

- Compass Surveying – Measures directions using magnetic bearings.

- Plane Table Surveying – Field plotting and measurement simultaneously.

- Levelling – Determines relative heights (vertical distances).

- Theodolite Surveying – Measures both horizontal and vertical angles.

- Tacheometric Surveying – Determines distances and elevations indirectly by staff readings.

- Photogrammetric / Aerial Surveying – Uses aerial photographs for mapping.

- EDM / Total Station Surveying – Uses electronic instruments for precise measurements.

- GPS Surveying – Uses satellite-based data for global positioning.

2. Principles of Surveying

- Working from Whole to Part:

  - Main control points established first with high precision.

  - Minor details (subdivisions) measured with less precision.

  - Prevents accumulation of errors.

- Location of a Point by Two Measurements:

  - Each point should be located by at least two independent measurements (e.g., by distance and angle, or by intersection of lines).

  - Ensures accuracy and cross-verification.

3. Chain Surveying

- Definition: Chain surveying is the simplest method of surveying in which linear measurements are taken with chains or tapes, and details are fixed by offsets.

- Applicability: Used for small, level, open areas with simple details (e.g., roads, small plots).

- Principle: The area is divided into well-conditioned triangles (sum of angles between 30°–120°) to ensure accurate plotting.

Main Components:

- Main stations: Prominent points controlling the framework.

- Main lines: Lines joining main stations.

- Check lines: Verify accuracy of framework.

- Tie lines: Connect subsidiary points for detailed plotting.

- Base line: Longest and most important line, measured carefully.

Procedure:

- Reconnaissance: Preliminary inspection of the area.

- Selection of stations and lines.

- Measurement of lines with chain/tape.

- Recording observations in field book.

- Plotting of triangles and offsets on drawing sheet.

4. Types of Chains

| Type | Length | No. of Links | Use / Remarks |
| --- | --- | --- | --- |
| Metric Chain | 20 m or 30 m | 100 or 150 | Standard for engineering surveys in India |
| Engineer’s Chain | 100 ft | 100 | Used for engineering work (older unit) |
| Gunter’s Chain | 66 ft | 100 | Used for land measurement and area in acres |
| Revenue Chain | 33 ft | 16 | Used for cadastral work and small-scale surveys |
| Surveyor’s Chain | 100 ft | 100 | Used in general purpose work |

- Construction:

  - Links made of galvanized steel wire (4 mm diameter).

  - Brass tallies at every 5 m or 10 links for quick reading.

  - Brass handles at each end marked with the total chain length.

5. Types of Tapes

| Type | Material & Length | Accuracy / Use |
| --- | --- | --- |
| Cloth / Linen Tape | 10–30 m | Rough measurements, prone to stretching. |
| Fibre Glass Tape | 20–50 m | Non-corrosive, flexible, moderate accuracy. |
| Steel Tape | 1–50 m | High accuracy, used for precise linear measurement. |
| Invar Tape | 30–100 m (nickel–steel alloy) | Very low thermal expansion; used for high-precision geodetic work. |

Corrections Applied:

- For temperature, pull, sag, slope, and incorrect length of tape.

6. Instruments Used in Chaining, Ranging, and Offsets

(a) Chaining Instruments

- Chain / Tape: For distance measurement.

- Arrows / Marking Pins: To mark end points while chaining.

- Pegs: Wooden stakes (15 mm square, 150 mm long) to mark stations.

- Ranging Rods: Painted poles (2 m or 3 m) for aligning straight lines.

- Plumb Bob: For vertical alignment when chaining on slopes.

(b) Ranging Instruments

Used to align intermediate points in a straight line between two end stations.

- Ranging Rod: Painted red-white or black-white bands (each 20 cm).

- Line Ranger: Optical device used for quick ranging of intermediate points.

- Ranging Poles: Similar to rods but heavier for permanent points.

- Direct Ranging: Used for short distances (clear line of sight).

- Indirect Ranging: Used when line of sight is obstructed (hill, tree, etc.).

(c) Instruments for Taking Offsets

Offsets are perpendicular or oblique distances measured from the survey line to details.

- Cross-Staff: Simple wooden or metallic instrument to set right angles.

  - Types: Open cross-staff, French cross-staff, Adjustable cross-staff.

- Optical Square: Uses mirrors to produce 90° reflection; more accurate than cross-staff.

- Prism Square: Modern form of optical square using prisms for right angle sighting.

- Tape / Chain: For measuring length of offsets.

- Clinometer: Measures slopes and angles for slope correction.

7. Field Precautions and Care of Instruments

- Check chain length before and after use.

- Remove kinks and clean chain after use.

- Ensure rods are vertical using a plumb bob.

- Avoid tightening screws excessively.

- Protect all instruments from dust, moisture, and vibration.

8. Summary

- Surveying involves measuring horizontal and vertical distances and angles to determine relative positions.

- Chain surveying is the foundation of all fieldwork, using chains, tapes, and simple optical devices.

- Classification and principles ensure systematic and accurate mapping.

- Modern instruments like EDMs, total stations, and GPS have enhanced precision, but the basics of chaining and ranging remain fundamental.